The Tennessee Eastman Simplified (TES) model can be found in (Ricker 1993). It is a simplification of the model described in (Downs & Vogel 1993), with only one process unit. This process unit is a combination of a reactor and a separator (see Figure 1). This unit has 2 input flows (feed 1 and feed 2) and two output flows (purge and stream 4).

Purge A+B+C Stream 4 D Reactor+Separator Feed 1 A+B+C Feed 2 A D A, B, C

Figure 1. Schematic diagram of the TES process

Ricker proposed different ways to control de TES process (Ricker 1993). In this virtual-lab is implemented the same multiloop control strategy that he proposes using 4 PI controllers (see Figure 3). The parameters of these controllers can be found in (Ricker 1993).

Figure 3. Multiloop control strategy

One of the control scenarios proposed by (Ricker 1993) has been simulated using this Simulink model. In this scenario, the setpoints of the first three loops were changed step-wise at t=0 to a product flow setpoint of 130 Kmol/hour, a pressure setpoint of 2850 KPa and the amount of A in the purge setpoint of 63%.

The time evolution (in hours) of the product flow (kmol/hour), pressure (KPa), amount of A in the purge (%) and the manipulated variables u1, u2 and u3 (%) obtained by simulating the Simulink model are shown in Figure 5. They can be compared to the figures shown in (Ricker 1993), showing a similar time evolution of the controlled variables. After 30 hours, all controlled variables are near their setpoints.